INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -
S.C.


[image: image1.wmf]  S.A.
________________________________________________________________________________________________________________________


	LP 900 C
Lapping Machine



[image: image2.png]



	The LP 900 C product belongs to the category of superfinishing machine for the parts with plane, plan-paralell or cylindrical surfaces.
The machine consists of two grinding wheels: inferior and superior. The parts are situated between the two grinding wheels and by combination of  the rotation motion of the disks and of the satellites (workingpiece rings) is obtained a motion which creates a permanent abrasive wear of the parts with lapping wheel.



TECHNOLOGIC  CAPABILITIES

	Lapping machine is destinated to precision working for achieving surfaces with high finishing degree.



TECHNICAL FEATURES
	- Lapping wheel external diameter



mm


900
- Rolling rings diameter




mm


303
- Superior disk stroke, max.




mm


200
- Superior disk speed





rot/min


41

- Inferior disk speed





rot/min


37

- Workingpiece ring drive speed



rot/min


40
- Superior disk drive power




kW


2,2 / 4

- Superior disk drive motor speed



rot/min


750 / 1500
- Inferior disk drive motor power



kW


6 / 9
- Inferior disk drive motor speed



rot/min


750 / 1500
- Machine weight





kg


4000






MACHINED PIECE


	INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -
ROMANIA - 030352, Bucharest 49, Bvd. Basarabia  250

Tel/ Fax:: (+4021) 255 17 70, mobile phone +40745 581 518

E - mail: office@simtex-mu.ro    http://www.simtex-mu.ro



[image: image3.wmf]


















20





30





50








_1169423310

