INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -

S.C.


[image: image1.wmf]  S.A.
__________________________________________________________________________________________________________________________
	SAMP - 01
Automatisation system with gantry handling and two conveyors for piece handling type drum at vertical lathe SV 1/5 CNC 


[image: image2.jpg]ki ;//77';
i ,.,{ s
R e

IR il

1
i)
'Y
i
3
1

T RS





	Automatisation system is composed by:

- receiving stocking conveyor

- receiving conveyor lift

- handling

- evacuation conveyor lift

- evacuation stocking lift

- control and driving equipment

- protection fence




TECHNOLOGICAL POSIBILITIES

	Automatisation system with gantry handling and two conveyors performs piece handling type drum at vetrical lathe SV 1/5 CNC.




TECHNICAL FEATURES

	- Conveyor:
· rolles diametre




     mm


     90

· superior plane level in rapport with flour 

     mm 

     cca. 300
· axes with rolles step 



     mm


     150
· transport speed 




     m/min 

     4
· electrical power




     kW


     0,63
· overall dimensions (L x W x H)


     mm


     2250x980x360

- Conveyor lift:

· conveyor transport speed



     m/min 

     3,5 
· platform lift - lower speed



     m/min 

     5
· overall dimensions (L x W x H)


     mm


     910x910x2210

- Handling:

· working cycle:sequential control with

speed reduction and stop at switch on frein
· number of units in automat cycle


     -


     3

· number of units in setting mode


     -


     1

· accuracy of repetability



     mm


     ± 3
· max. load on arm

 


     daN 

     80
· horizontal stroke, max.
 


     mm


     6000
· vertical stroke, max. 



     mm 

     1000
· transport speed (cont. ajustable), max.

     m/min

     18
· overall dimensions (L x W x H)


     mm


     8200x85x4350
- Grippere:

· dies stroke, max.




     mm


     40
· lifting capacity (with shape 


     daN 

     80
     prehension)

· thinghing force, max.



     daN

     1000
· overall dimensions (L x W x H)


     mm


     305x215x270



	INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -

ROMANIA - 030352, Bucharest 49, Bvd. Basarabia  250

Tel/ Fax:: (+4021) 255 17 70, mobile phone +40745 581 518

E - mail: office@simtex-mu.ro    http://www.simtex-mu.ro



[image: image3.wmf]
_1169423357

_1169423310

