INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -

S.C.


[image: image1.wmf]  S.A.
__________________________________________________________________________________________________________________________
	SAPF - 01
Feeding system for tirfon head forging press




[image: image2.jpg]



	Feeding system for tirfon head forging press is composed by:

- semi-product clamping mechanism

- transfer driving mechanism

- transfer mechanism with cross slides

- assembly receive tirfon half-product from oven 

- pneumatical installation

- electrical installation




TECHNOLOGICAL POSIBILITIES

	Feeding system performs half-product piece transport from electric oven at press and the introduction in fixed half-stamping die.



TECHNICAL FEATURES

	- Machining piece dimensions:
· diametre: - min.




     mm


     21,6
  - max.




     mm


     21,9
· length: - min.




     mm


     190  

- max.




     mm


     240  

- Bore diametre in steps of

  fixed half-stamping die 

· in diametre





     mm


     23,8  
· depth diametre




     mm


     22,4  
- Working cycle




     -


     automat

- Working tact





     sec


     4,5 

- Working stroke:

· longitudinal slide




     mm


     1200

· transversal slide




     mm


     125

· tightening-loosing piece



     mm


     25

- Moving speed:

· longitudinal slide plateau



     m/min.

     48

· transversal slide plateau



     m/min.

     15

· tightening-loosing piece



     m/min.

     3

- Air preassure,net, min.



     bar


     6 

- Overall dimensions (L x W x H)


     mm


     1812x1100x2480




	INDUSTRIAL SYSTEMS OF MACHINES, TECHNOLOGIES, MANUFACTURING, RESEARCH, EXPORT

- joint stock company -

ROMANIA - 030352, Bucharest 49, Bvd. Basarabia  250

Tel/ Fax:: (+4021) 255 17 70, mobile phone +40745 581 518

E - mail: office@simtex-mu.ro    http://www.simtex-mu.ro



[image: image3.wmf]
_1169423357

_1169423310

